Lab 09

Design of a Light switch

Spring 2025

Submitted by: Mohsin Sajjad

Registration No: 22pwsce2149

Class Section: A

“On my honor, as student of University of Engineering and Technology, I have neither given nor received unauthorized assistance on this academic work.”

Student Signature: ______________

Submitted to:

Engr. Faheem Jan

Month Day, Year (11 05, 2025)

Department of Computer Systems Engineering

University of Engineering and Technology, Peshawar

Design of a Light switch

Objective:

Build a light switch controller such that when the push button is pressed the light if “off” turns “on” and if “on” turns “off”.

Block Diagram:

CODE:

module light_switch (

input clk,

input rst,

input button,

output reg led

);

parameter S0 = 2'b00; // LED off

parameter S1 = 2'b01; // LED on

reg [1:0] state, next_state;

// Sequential block

always @(posedge clk or posedge rst) begin

if (rst) begin

state <= S0;

led <= 0;

end else begin

state <= next_state;

case (next_state)

S0: led <= 0;

S1: led <= 1;

default: led <= 0;

endcase

end

end

// Combinational block

always @(*) begin

case(state)

S0: next_state = (button) ? S1 : S0;

S1: next_state = (button) ? S0 : S1;

default: next_state = S0;

endcase

end

endmodule

// Clock Divider

module Clock_Divider (

input clock_in,

output reg clock_out = 0

);

reg [27:0] counter = 28'd0;

parameter DIVISOR = 28'd100000000;

always @(posedge clock_in) begin

if (counter == DIVISOR - 1) begin

counter <= 0;

clock_out <= ~clock_out;

end else begin

counter <= counter + 1;

end

end

endmodule

// D Flip-Flop

module df1 (

output reg q,

input d,

input clk,

input rst

);

always @(posedge clk or negedge rst) begin

if (!rst)

q <= 1'b0;

else

q <= d;

end

endmodule

// Synchronizer (2 DFFs in series)

module Synchronizer (

output sb,

input d,

input clk,

input rst

);

wire q;

df1 inst1(q, d, clk, rst);

df1 inst2(sb, q, clk, rst);

endmodule

// Level to Pulse Generator

module level_to_pulse (

input wire clk,

input wire rst,

input wire level,

output wire pulse

);

wire q;

df1 dff (

.q(q),

.d(level),

.clk(clk),

.rst(rst)

);

assign pulse = level & ~q; // Rising edge detection

endmodule

// Top Module

module top (

input wire btn,       // Raw button input

input wire RST,       // Asynchronous reset

input wire clk,       // System clock

output wire led       // Output LED

);

wire sync_button;     // Synchronized button signal

wire slow_clock;      // Divided (slow) clock

wire pulse_button;    // One-cycle pulse on button press

// Clock Divider to reduce system clock frequency

Clock_Divider c1 (

.clock_in(clk),

.clock_out(slow_clock)

);

// Synchronize button input to slow clock

Synchronizer s1 (

.clk(slow_clock),

.d(btn),

.rst(RST),

.sb(sync_button)

);

// Convert level-based button press to one-clock-cycle pulse

level_to_pulse l1 (

.clk(slow_clock),

.rst(RST),

.level(sync_button),

.pulse(pulse_button)

);

// Light switch FSM

light_switch l2 (

.clk(slow_clock),

.rst(RST),

.button(pulse_button),

.led(led)

);

endmodule

Output:

Conclusion:

This Verilog project creates a reliable light switch using an FSM. A clock divider slows down the system clock. Button signals are synchronized and turned into short pulses. The FSM changes the LED state with each button press. This setup ensures smooth LED control without glitches.